1. Hardware environment

| mode | Device |
| --- | --- |
| CAP_HDMIRX_MODE | Source is HDMI RX (IT6801) |
| CAP_MODULE_MODE | Source is sensor |
| CAP_DELAYWINDOWS_MODE | Scan target board delay window |
| Target board:EVB977 144PIN |  |

2.Choose front-end device

2.1 choose sensor (sensor on ,hdmi off)

- sensor enable

- sensor gpio

2.2 choose hdmirx(hdmi on ,sensor off)

- Hdmirx enable

3.Load capture init define:

根據target board & source來做設定:

3.1如果要新增sensor就要新增一個 (xxx.txt) file 至path: sdk\target\defcap\

4.IIC Setting

5.Capture delay test

Use CAP_DELAYWINDOWS_MODE run

show delay value = 0 ~ 15  stage , stable or unstable

example:

12 13 14 15 0 1 2 3 4 5 6 7  is stable. the median is 1 or 2 , 1 or 2 is best.

Set 1 or 2 in the VIDEO_TALBE ClkDelay item(according to your source timing)

sdk\include\capture\video_device_talbe.h

static CAP_NORMAL_TIMINFO_TABLE VIDEO_TABLE []={

{ Index, HActive, VActive,…,…, ClkDelay },

}

6.Run Normal Test

7.Error Status

Error code table:

| Bits | Type | description |
| --- | --- | --- |
| 0 | Read Only | 1:Hsync Stable 0:Hsync Unstable |
| 1 | Read Only | 1:Vsync Stable 0:Vsync Unstable |
| 2 | Read Only | 1:DE Stable(X) 0:DE Unstable(X) |
| 3 | Read Only | 1:DE Stable(Y) 0:DE Unstable(Y) |
| 11:8 | Read Only | Error code [1]:Hsync loss [2]:Vsync loss [3]:DE loss [4]:frame end error [5]:capture overflow [7]:frame rate change [8]:time out |

Example:

error code is [3] DE Loss.

9.Show on the Panel

Set Kconfig Screen. LCD enable.